EEG Pre-processing Pipeline for ERP analysis

- Load .cnt file

  - Edit  Channel locations  When the autodetect window shows up, click “Cancel”. àThe following window will show up, note there are only channel labels but no coordinates.

  - Click “Read locations” on the bottom left

  - Select “NeoNet_64_chanloc.ced”, from whichever folder you saved the file.

  - Check M1 (channel 33), M2 (43), EOG

- Bandpass filter (bpf0.1-5) (not used)

  - Tools  Filter  Basic FIR Filter (new, default)  Bandpass filter 0.1 – 5 Hz

  - Tools  Filter  Basic FIR Filter (new, default)  Notch filter 59-61 Hz

- Bandpass filter(bpf0.1-30)

  - ERPLAB  Filter & Frequency Tools  Filters for EEG data  Bandpass filter 0.1 – 30 Hz

  - ERPLAB  Filter & Frequency Tools  Filters for EEG data  Notch filter 60 Hz

- Re-reference to mastoids: (rerefM)

  - Tools  Re-reference  Re-reference data to channel(s):  M1 M2

  - Exclude channel indices (EMG, EOG) à select VEO and HEO [EOGs should not be re-referenced to mastoids]

- Automatic Artifact Removal (noEMG)

  - Muscle artifacts:

    - Tools à Artifact removal using AAR à EMG Removal à Using BSS à both window lengths 5

- Creating an EventList (elist)

  - ERPLAB  EventList  Create an EEG EVENTLIST

  - Keep boundary events, and all event names should be numerical

- Assigning trials to bins (binsS/binsR)

  - Create Bin-Descriptor Files

  - Example:

    - bin <number>

    - Name of bin

    - .{1;2}{3}

    - . is the anchor point, ; means the two events are equivalent, each bracket must be followed in order to be put in this bin

  - ERPLAB  Assign bins (BINLISTER)

- Extract bin-based epochs (be)

  - ERPLAB  Extract bin-based epochs

  - Time range: [-500 3000] for stim event, [-500 0] for baseline, [-2000 500] for response event, [-2000 -1500] for baseline

- Artifact Detection (ar)

  - Manually mark epochs for rejection in Tools  Inspect/reject data by eye  Check add to previously marked rejections, do not check reject marked trials highlight all bad trials  UPDATE MARK

  - Note marked trials

  - Synchronize to EVENTLIST by ERPLAB  Artifact Detection  Synchronize Artifact info in EEG and EVENTLIST  Transfer EEG.reject artifact marks to EVENTLIST

- ICA (icaLabeled/icaRejected)

  - Tools  Decompose data by ICA  Runica – select channels

  - Save marked .set file

  - Tools  Classify components using ICLabel  Label components

  - Tools  Reject data using ICA  reject components by map

  - Reject components

- Create Averaged ERPset

  - ERPLAB  Compute averaged ERPs

  - Save ERPSET

- Create Difference Channels

  - ERPLAB  ERP operations  ERP channel operations

- Create difference ERPs

  - ERPLAB  ERP operations  Append ERPsets  Apend block 1 and 2

  - ERPLAB  ERP operations  ERP bin operations  subtract block 1 from block 2

ERPLAB tutorial: https://github.com/lucklab/erplab/wiki/Tutorial-1-EEG-to-ERPset