EEG Pre-processing Pipeline for modular and nodal network analysis

- Load .cnt file

- Edit  Channel locations (** Use the autodetect one from eeglab)

  - Check M1 (channel 33), M2 (43), EOG

- Bandpass filter

  - 0.3 – 50 Hz

Program: bpfEEG_prep.m

Functions: bp_filter.m, do_fft.m

After running first section of bpfEEG_prep.m, use do_fft to check whether bpf was performed correctly.

Then run the second section to save .set file

  - Load the saved file into eeglab

  - Tools  Filter  Basic FIR Filter (new, default)  Notch filter 59-61 Hz

- Re-reference to mastoids:

  - Tools  Re-reference  select re-reference data to channel(s)  M1, M2

- Automatic Artifact Removal

  - Eye artifacts:

    - Artifact removal using AAR  EOG Removal  Using BSS  window length 256  EOG channel indexes 63 64

  - Muscle artifact:

    - Artifact removal using AAR  EMG Removal using BSS  window length 5

- Epoch

  - Marker: 255

  - Time window: [-1 2]

  - Click cancel when “remove base” window pops up

- ICA

  - Runica – select channels (1-62)

  - Reject data using ICA  reject components by map

  - Save marked .set file

  - Reject components

NOTE: Steps 4 and 5 are subject to change based on data quality