EEG Pre-processing Pipeline for modular and nodal network analysis

- Load .cnt file (Luke’s data: neuroscan .CNT)

- Edit à Channel locations (** Use the autodetect one from eeglab)

à in the pop-up window, select “…” to upload channel location file “SynAmps2 Quik-Cap64.DAT” from “Channel Location Files -SynAmps2-Quik-Cap64” folder

  - Check M1, M2, EOG

- Bandpass filter

  - 0.3 – 50 Hz

Program: bpfEEG_prep.m

Functions: bp_filter.m, do_fft.m

After running first section of bpfEEG_prep.m, use do_fft to check whether bpf was performed correctly.

Then run the second section to save .set file

  - Load the saved file into eeglab

  - Tools à Filter à Basic FIR Filter (new, default) à Notch filter 59-61 Hz

- Remove mastoid channels:

  - Edit à select data à chan range à choose M1, M2 àcheck “remove these”

- Epoch

  - Marker: 255

  - Time window: [-1 2]

  - Click cancel when “remove base” window pops up

- ICA

  - Runica – select channels (1-62)

  - Reject data using ICA à reject components by map

  - Save marked .set file

  - Reject components

NOTE: Steps 4 and 5 are subject to change based on data quality

Neuroscan website: http://compumedicsneuroscan.com/scan-acquire-configuration-files/